SPILL AND LEAK CLEANUP INFO			SOP122

Any spill that involves diesel, gasoline or other hazardous chemicals.

1. Over 50 gals.

2. Any amount that enters a stream, creek or river.

3. Any amount that enters sewer system.

Must be reported to the Indiana Department Environmental Management.

1-888-233-7745 24 hour emergency numbers

- Cleanup Information for Vincennes Twp. Fire Department

Any cleanup involving more than 200 Lbs. of absorbing materials must be handled by a cleanup company, for disposal.

Any cleanup involving less than 200 Lbs. (per incident) of absorbing materials going into the fire station dumpster must be recorded in the log book and the dispatch computer run close out data, for tracking and follow-up reports.

Any materials applied at a spill scene must be picked up. Sand can only be used for traction on roadways, not to be used for cleanup. Sand may be used for diking, in ditches or creeks before Cleanup Company arrives.

On a large spill on a roadway VTFD can block the road and wait for the cleanup company (or), block the road and apply absorbal and the cleanup company will pick up and dispose of all materials used.

Also advise Cleanup Company of all supplies VTFD used and they will replace the supplies at the scene, or deliver them later.

- Cleanup Companies for Vincennes Twp. Fire Department

At the scene, ask the person or company involved if they have a pre-arranged agreement for a cleanup company, if so get a name and phone number.

Call that company and get an estimated time for arrival at the scene. They may also need to be briefed on the spill so they will know what equipment to respond with. Also get a cell phone number for the person in charge of the cleanup company enroute to the scene.

If no cleanup company has been pre-arranged, you may provide the person or company involved with the numbers of these companies.

Summit Environmental			812-421-1744

877-421-1744

American Enviro Services Inc.		812-858-3151

800-421-7646

VTFD HAZMAT LAB                                                      PAGE 42                                              UPDATE 11-21-01